ESP32 LASER TAG SYSTEM

Full Build Guide & System Architecture

4-Player P2P Authenticated Laser Tag

| Players | Communication | Hit Detection | Security |
| --- | --- | --- | --- |
| 4 players (expandable) | ESP-NOW P2P | IR receivers | ECDH session keys HMAC-signed packets |

1. Project Overview	4

1.1 System Overview	4

1.2 Build Phases	4

2. Components & Cost	5

2.1 Per Player Unit (×4)	5

2.2 One-Off Costs	5

2.3 Buy in Stages	6

3. Hardware Architecture	7

3.1 System Layout	7

3.2 Gun–Vest Cord	7

3.3 IR Receiver Placement	7

3.4 IR Transmitter (Gun Barrel)	8

3.5 WS2812B LED Chain	8

3.6 Buttons and Controls	8

3.7 Battery	8

4. IR Protocol	9

4.1 Physical Layer	1

4.2 Packet Structure (32 bits)	1

4.3 Weapon Modes	1

4.4 Friendly Fire	1

4.5 Nonce & Replay Prevention	1

5. ESP-NOW Network Layer	1

5.1 Characteristics	1

5.2 Node IDs	1

5.3 Packet Types	1

5.4 Game State Table	1

5.5 Sync Rules	1

6. Hit Confirmation Protocol	1

6.1 The Problem	1

6.2 Flow	1

6.3 What Each Actor Can and Cannot Fake	1

6.4 Pending Entry Expiry	1

7. Cryptography & Authentication	1

7.1 Why Bother	1

7.2 ECDH Key Exchange (Pre-Game)	1

7.3 Keys Live in RAM Only	1

7.4 HMAC Signing	1

7.5 Rebooted Node Behaviour	1

7.6 Packet Validation Checklist	1

8. Game Logic	1

8.1 Game States	1

8.2 Lobby Phase	1

8.3 Starting HP & Teams	1

8.4 Win Condition	1

8.5 Death	1

8.6 Reset Vote	1

9. Display & Feedback	1

9.1 OLED Display Layout	1

9.2 RGB LEDs (WS2812B)	1

9.3 Vibration Motor	1

9.4 Buzzer (Passive)	1

10. Silent Mode	1

11. Build Order	1

Phase 1: Single Unit Hardware	1

Phase 2: Networking & Game Logic	1

Phase 3: Authentication	1

12. Arduino IDE Setup	1

12.1 Board and Core	1

12.2 Libraries Required	1

12.3 Key ESP-NOW Notes for Core v3.x	1

13. Testing Checklist	1

Single Unit	1

Two Units	1

Four Units: Full Game	1

14. Anti-Cheat Summary	1

15. Future Extensions	1

15.1 Repeater Network	1

15.2 Additional Weapon Modes	1

15.3 Spectator Node	1

15.4 Post-Game Stats	1

# 1. Project Overview

This guide covers the complete build of a 4-player ESP32-based laser tag system with peer-to-peer anti-cheat consensus, ECDH session key authentication, IR-based hit detection, and full game logic.

💡 The system is intentionally overengineered. It uses the same cryptographic primitives as TLS and Signal, applied to a laser tag game.

## 1.1 System Overview

- 4 player units, each with a gun and vest

- IR hit detection with 5 receivers per player

- ESP-NOW peer-to-peer mesh for game state synchronisation

- Two-factor hit confirmation: IR packet corroborated by ESP-NOW

- ECDH session keys with cryptographic eviction on power cycle

- OLED display, RGB LEDs, vibration motor, and buzzer per unit

- 4 teams, colour-coded; last team standing wins

## 1.2 Build Phases

Build in three phases. Get hardware working first, then add networking and game logic, then layer authentication on top.

| Phase | Goal | Estimated Time |
| --- | --- | --- |
| Phase 1 | Hardware: all peripherals working on one unit | 10–16 hours |
| Phase 2 | Networking: ESP-NOW, game logic, win conditions | 14–24 hours |
| Phase 3 | Auth: ECDH keys, HMAC signing, anti-cheat | 11–17 hours |

# 2. Components & Cost

## 2.1 Per Player Unit (×4)

| Component | Notes | Cost (AUD) |
| --- | --- | --- |
| ESP32 DevKit | AliExpress: allow 3–4 weeks | $5–8 |
| TSOP38238 IR receiver ×5 | Buy a 20-pack, 38kHz, one per hit zone | $1–2 |
| High power IR LED ×2 | TSAL6400 or SFH4545, gun + spare | $1–2 |
| 2N2222 NPN transistor ×2 | IR LED drive circuit | $0.50 |
| SSD1306 OLED 128×64 | I2C, straightforward with ESP32 | $3–5 |
| WS2812B LED strip segment | Chainable; one data line drives all | $1–2 |
| Vibration motor ×1 | Coin type, 3V | $1 |
| Passive buzzer ×1 | For tunes: active buzzers are single-tone only | $0.50 |
| 18650 battery ×1 | Samsung 25R or similar, 2500mAh | $5–8 |
| TP4056 charge module ×1 | USB-C variant recommended | $1 |
| Toggle switch + tactile buttons ×4 | Power, reload, team select, silent mode | $1 |
| Wiring, resistors, stripboard | Prototyping and final build | $2–3 |
| Per unit total |  | $22–33 |

## 2.2 One-Off Costs

| Item | Notes | Cost (AUD) |
| --- | --- | --- |
| Spare ESP32s ×2 | Recommendation: failures during development are expected | $10–16 |
| Bulk IR components | Covers whole project | $3 |
| Soldering consumables | Flux, solder, wick | $5 |
| Jumper wires and breadboard | Prototyping phase | $3 |
| One-off total |  | $21–27 |

Total project cost: approximately $109–159 AUD for all 4 units.

## 2.3 Buy in Stages

Purchasing in stages reduces waste if hardware changes are needed during development.

### Stage 1: Prototype

- ESP32s, breadboard, IR LEDs, TSOP receivers, OLED, buzzer, resistors

- Under $30; covers all Phase 1 prototyping

### Stage 2: Build Out

- After the prototype works: batteries, charge modules, stripboard, switches, WS2812B LEDs, vibration motors

### Stage 3: Multiply

- Bulk components for all 4 units once a single unit is proven

# 3. Hardware Architecture

## 3.1 System Layout

One ESP32 per player, located in the gun. The vest connects to the gun via a 7-wire cord. All interactive components (display, buttons, motor, buzzer, IR emitter) are in or on the gun. The vest and sensors carry on,ly IR receivers and WS2812B LEDs.

## 3.2 Gun–Vest Cord

Use thin 8-core flexible cable (headphone extension or similar). 7 wires are used; 1 is spare.

| Wire | Purpose |
| --- | --- |
| VCC | Power to vest components from gun-side regulator |
| GND | Common ground |
| IR_FRONT | TSOP38238 signal: front of vest |
| IR_BACK | TSOP38238 signal: back of vest |
| IR_LEFT | TSOP38238 signal: left shoulder |
| IR_RIGHT | TSOP38238 signal: right shoulder |
| WS2812B_DATA | Single data line driving all RGB LEDs on vest and sensors |

💡 Use a strain relief at both ends so pulling on the cord does not stress the solder joints.

## 3.3 IR Receiver Placement

All 5 receivers feed into the ESP32 as separate GPIO inputs. Any one triggering counts as a hit.

- Front: centre of chest

- Back: centre of back

- Left: left shoulder

- Right: right shoulder

- Gun: on the body of the gun: getting shot in the gun counts as a hit, preventing corner-peeking

bool hitDetected() {

return !digitalRead(IR_FRONT) || !digitalRead(IR_BACK) ||

!digitalRead(IR_LEFT)  || !digitalRead(IR_RIGHT) ||

!digitalRead(IR_GUN);

}

## 3.4 IR Transmitter (Gun Barrel)

Collimate the IR LED inside a short tube or barrel to focus the beam and increase range. Drive the LED via a 2N2222 transistor: do not drive directly from the ESP32 GPIO pin (max 40mA, insufficient for range).

| Setup | Expected Range (Outdoors, Sun) |
| --- | --- |
| Standard IR LED, no collimation | 5–8m |
| High power IR LED (SFH4545) | 8–15m |
| High power + collimating tube | 15–20m |

⚠️ IR noise floor rises sharply outdoors. Plan for 8–12m realistic engagement range.

## 3.5 WS2812B LED Chain

All RGB LEDs on a player unit are chained on a single data line. Chain order: gun LEDs → vest LED → sensor housing LEDs. This allows team colour, hit flash, silent mode, and death effects to all be driven from one GPIO pin.

## 3.6 Buttons and Controls

| Control | Location | Function |
| --- | --- | --- |
| Power switch | Gun | Cuts VCC to whole unit |
| Reload button | Gun | Refills AR ammo after 2s delay |
| Team cycle button | Gun | Steps through team colours in lobby |
| Silent mode button | Gun | Activates 2-minute stealth mode |
| Display VCC switch | Gun | Physically cuts power to OLED |
| Reset vote button | Gun | Casts vote to reset/restart round |

## 3.7 Battery

Single Samsung 25R 18650 (2500mAh) per unit. At approximately 300mA average draw (ESP32 active + display + LEDs + occasional motor), this gives roughly 8 hours of runtime. Two cells in parallel gives 16 hours: one cell is sufficient; use two only if weight is not a concern.

💡 Use a TP4056 with protection circuit (labelled 1S with B+/B- pads). Power the ESP32 from the battery via a small 3.3V LDO, or use the DevKit onboard regulator if running from 5V USB out of the TP4056.

\4. IR Protocol

## 4.1 Physical Layer

- 38kHz carrier frequency matches TSOP38238 filter, rejects ambient light

- NEC-style or custom encoding: custom is recommended as both ends are under project control

- Packet duration should be short enough to complete during movement

## 4.2 Packet Structure (32 bits)

Every IR packet transmitted by the gun contains exactly 32 bits:

| Field | Bits | Notes |
| --- | --- | --- |
| shooterID | 4 | Player 0–3; up to 16 players supported |
| nonce | 22 | Random value, ~4M possibilities; collision negligible at 4 players |
| damage | 4 | Weapon damage value, maps to weapon mode |
| checksum | 2 | XOR of all above fields; catches corrupt packets |

⚠️ A corrupt packet with a bad checksum is silently discarded.

## 4.3 Weapon Modes

The potentiometer on the gun maps to a weapon mode at the moment of firing. The damage value in the IR packet reflects the selected weapon.

| Mode | Damage | Ammo | Fire Rate | Display |
| --- | --- | --- | --- | --- |
| Pistol | 2 | Unlimited | 300ms cooldown | PISTOL |
| AR | 4 | 30 rounds | 150ms cooldown | AR |

💡 AR ammo refills on reload button press after a 2-second delay. The display shows current ammo count when in AR mode.

## 4.4 Friendly Fire

When a hit is received, the victim checks the shooterID in the IR packet. If it matches a team member, the hit is silently ignored: no HP change, no feedback, no ESP-NOW broadcast.

## 4.5 Nonce & Replay Prevention

The 22-bit nonce is a random value generated at the moment of firing. It is embedded in the IR packet and also broadcast over ESP-NOW by the shooter. This nonce is the key to two-factor hit confirmation (see Section 6).

- Maintain a used-nonce circular log of 32 entries on each node

- Reject any nonce already seen to prevent replay attacks

# 5. ESP-NOW Network Layer

## 5.1 Characteristics

- Single-hop broadcast: no mesh, no routing

- Range: approximately 200m open air, far exceeding the IR engagement range of 10–15m

- If two players can shoot each other with IR, their ESP32s can communicate directly

- No internet, no router, no server required

## 5.2 Node IDs

Node IDs are assigned at game start deterministically: no coordination required:

// Sort all MAC addresses seen during lobby phase.

// Each node's index in that sorted list = its playerID (0–3).

The same physical device always gets the same ID when the same set of devices is present.

## 5.3 Packet Types

| Packet Type | Direction | Purpose |
| --- | --- | --- |
| PUBKEY_BROADCAST | Broadcast | Share public key during pre-game key exchange |
| TEAM_SELECT | Broadcast | Announce team colour choice in lobby |
| READY_VOTE | Broadcast | Signal ready to start; game starts on unanimous vote |
| HIT_CLAIM | Broadcast | Victim announces they were hit, includes all 32 IR bits |
| SHOT_CONFIRM | Broadcast | Shooter confirms they fired nonce X, only after seeing matching HIT_CLAIM |
| SYNC_REQUEST | Broadcast | Request full game state table from peers |
| SYNC_RESPONSE | Unicast | Full table response to a sync request |
| RESET_VOTE | Broadcast | Vote to reset and start a new round |

## 5.4 Game State Table

Each node maintains a fixed table of 10 slots stored in SRAM. Slots 0–3 represent HP for players 0–3.

struct Slot {

uint8_t  ownerID;   // which player owns this slot

int32_t  value;     // current HP

uint32_t version;   // increments on every legitimate change

};

## 5.5 Sync Rules

- On boot: broadcast SYNC_REQUEST, collect responses from majority, adopt result

- Periodic sync every 30 seconds during game to catch drift

- Per slot: highest version number wins

- Version tie: lowest HP wins

- HP can only decrease: positive delta packets are rejected by all nodes

- A rebooted node starts at version 0 and cannot override stored state on other nodes

# 6. Hit Confirmation Protocol

## 6.1 The Problem

IR hit detection is local and unwitnessed. When a player is hit, only their own vest detects it. If the hit player simply broadcasts their new HP, the other nodes: who still hold the old HP: can outvote them via consensus and revert the hit, causing an accidental revive.

The solution is two-factor confirmation: the hit must be corroborated by both the victim (who received IR) and the shooter (who fired it).

## 6.2 Flow

- Player 2 fires gun. IR packet transmitted: { shooterID: 2, teamID: 1, nonce: 0xA3F7, damage: 4 }

- Player 1 vest receives IR hit. Checks teamID: not same team: valid hit.

- Player 1 broadcasts HIT_CLAIM: { victimID: 1, shooterID: 2, nonce: 0xA3F7, delta: -4, newHP: 16, version: 2, hmac }

- Player 1 retransmits HIT_CLAIM 2–3 times with 50ms spacing to ensure shooter receives it. Used-nonce log prevents processing the same hit twice.

- Player 2 sees HIT_CLAIM with nonce 0xA3F7 in their incoming packet log. This matches a nonce they recently fired.

- Player 2 broadcasts SHOT_CONFIRM: { shooterID: 2, nonce: 0xA3F7, hmac }

- All nodes receive both HIT_CLAIM and SHOT_CONFIRM for the same nonce. Damage is committed to the table.

## 6.3 What Each Actor Can and Cannot Fake

| Scenario | Result |
| --- | --- |
| Legitimate hit: both packets arrive | Damage committed |
| Victim fakes being hit by Player 2 | Player 2 never fired; no SHOT_CONFIRM; pending expires; rejected |
| Shooter spams fake SHOT_CONFIRMs | No matching HIT_CLAIM from victim; pending expires; rejected |
| IR packet genuinely missed by vest | No HIT_CLAIM generated; no damage. Correct behaviour. |

## 6.4 Pending Entry Expiry

Nodes hold unmatched HIT_CLAIMs and SHOT_CONFIRMs in a pending buffer. If the matching half does not arrive within 2 seconds, the pending entry expires and no damage is applied.

# 7. Cryptography & Authentication

💡 This section can be skipped entirely for Phase 1 and Phase 2. Implement it in Phase 3 once the game is working.

## 7.1 Why Bother

Without auth, a technically inclined player could send forged ESP-NOW packets to manipulate game state. The ECDH system prevents this with no server and no pre-shared secrets. It also means a rebooted node is cryptographically locked out until a new round starts.

## 7.2 ECDH Key Exchange (Pre-Game)

ECDH (Elliptic Curve Diffie-Hellman) allows two nodes to derive a shared secret without ever transmitting it. Use the uECC library, which runs comfortably on ESP32.

- All nodes power on and enter lobby phase.

- Each node generates an ephemeral keypair: privateKey (never leaves device) and publicKey (broadcast to all).

- Each node collects publicKeys from all other nodes.

- Each node derives pairwise shared secrets locally using their own privateKey and each peer's publicKey.

- Game start vote is cast. Key exchange locks: no new publicKeys accepted after this point.

- All subsequent packets are HMAC-signed using the relevant pairwise shared secret.

uECC_set_rng(esp32RNG);  // use ESP32 hardware RNG

uECC_make_key(publicKey, privateKey, uECC_secp256r1());

uECC_shared_secret(theirPublicKey, privateKey, sharedSecret, uECC_secp256r1());

## 7.3 Keys Live in RAM Only

Private keys and derived shared secrets are never written to Preferences or flash: they exist only in RAM. A power cycle wipes them. This is intentional: a rebooted node cannot reconstruct its session keys and therefore cannot produce valid HMAC signatures. Its packets are rejected by all other nodes.

## 7.4 HMAC Signing

Each packet includes an HMAC field computed over the packet contents using the pairwise shared secret. For this threat model, a keyed CRC32 is sufficient:

uint32_t hmac = CRC32(senderID ^ delta ^ newHP ^ version ^ *(uint32_t*)sharedSecret);

💡 For stronger security, use SHA256-HMAC. The ESP32 has hardware SHA acceleration so this adds negligible overhead.

## 7.5 Rebooted Node Behaviour

- Has no private key and cannot derive shared secrets

- Cannot produce valid HMAC: all packets are rejected by other nodes

- Can still receive and store packets from others

- Broadcasts SYNC_REQUEST; gets current HP restored from other nodes

- Is effectively read-only until the next round

- Client side: all LEDs flash and buzzer sounds to indicate the eviction

## 7.6 Packet Validation Checklist

Every node applies all of these checks to every received packet, in order:

- HMAC valid using pairwise shared secret

- Delta is negative

- Version is exactly stored version + 1

- currentHP + delta == newHP

- Nonce not in used-nonce log

- Both HIT_CLAIM and SHOT_CONFIRM received for nonce

# 8. Game Logic

## 8.1 Game States

| State | Description |
| --- | --- |
| LOBBY | Players cycle team colours, key exchange happens, waiting for unanimous ready vote |
| STARTING | All ready votes received; countdown begins |
| IN_GAME | Active play: hit detection, ESP-NOW active, HP tracked |
| ROUND_OVER | Win condition met: display result, waiting for reset vote |
| RESETTING | Unanimous reset vote received: clear state, return to LOBBY |

## 8.2 Lobby Phase

- All nodes power on and broadcast their MAC address.

- Node IDs assigned: sort MAC addresses; index = nodeID.

- ECDH key exchange runs: public keys broadcast and collected.

- Each player presses team cycle button to step through Red → Blue → Green → Yellow.

- TEAM_SELECT packets broadcast on each change; all displays show current team assignments.

- Player presses ready button: READY_VOTE broadcast.

- Once all 4 nodes have sent READY_VOTE, game transitions to STARTING.

## 8.3 Starting HP & Teams

- Starting HP: 20

- 4 teams: Red, Blue, Green, Yellow

- Team sizes are flexible: 2v2, 1v1v1v1, and 1v3 all work; the win condition handles all cases

- Friendly fire is off: if shooterID in IR packet matches team in data table, the hit is silently ignored

## 8.4 Win Condition

Last team standing. After every HP change, all nodes evaluate:

// Collect all living players (HP > 0)

// If all living players share the same teamID → that team wins

// If only one player remains → they win

Win is broadcast to all nodes. All displays show WIN or LOSE. The winner plays a tune; dead players and losers play a different tune.

## 8.5 Death

- HP reaches 0: player is dead for the rest of the round

- Display shows GAME OVER

- Vibration motor fires with a longer pattern than a normal hit

- All LEDs turn off (team colour extinguished)

- Dead node stays on network: participates in consensus as a witness but cannot deal or receive scoring hits

- Dead node can still cast a reset vote

## 8.6 Reset Vote

Any player presses the reset button to cast a RESET_VOTE. Once all nodes: including dead players: have voted, the round resets:

- All HP restored to 20

- Version counters reset

- Used-nonce logs cleared

- New ECDH key exchange begins

- Game returns to LOBBY state

⚠️ All nodes must vote, not just survivors. This prevents one surviving player from force-resetting when another has not accepted defeat.

# 9. Display & Feedback

## 9.1 OLED Display Layout

| Game State | Display Content |
| --- | --- |
| LOBBY | Node ID, current team colour, other players' team selections, READY status |
| IN_GAME (Pistol) | HP bar, PISTOL label, team status (X alive) |
| IN_GAME (AR) | HP bar, AR label, ammo count (e.g. 24/30), team status |
| Hit received | HP bar updates, brief HIT flash |
| Kill confirmed | KILL!: shown for 1 second then returns to normal |
| Dead | GAME OVER: stays until reset |
| Round won | YOU WIN: stays until reset |
| Round lost | YOU LOSE: stays until reset |
| Silent mode | Display dims to minimum brightness |

## 9.2 RGB LEDs (WS2812B)

| Condition | LED Behaviour |
| --- | --- |
| Lobby: team not selected | Slow white pulse |
| Lobby: team selected | Solid team colour |
| In game: alive | Solid team colour |
| Hit received | Brief white flash, returns to team colour |
| Dead | LEDs off |
| Silent mode active | All LEDs off |
| Cryptographic eviction (reboot cheat) | Rapid red flash |
| Round won | Colour chase animation |

## 9.3 Vibration Motor

| Event | Pattern |
| --- | --- |
| Hit received | Single short pulse (~100ms) |
| Kill confirmed on gun | Two short pulses |
| Death | Long pulse (~500ms) |
| Silent mode activate | Two short pulses |
| Silent mode deactivate | Two short pulses |
| Game won | Three short pulses |

## 9.4 Buzzer (Passive)

| Event | Sound |
| --- | --- |
| Silent mode activate | Short descending two-note tone |
| Silent mode deactivate | Short ascending two-note tone |
| Death | Descending multi-note tune |
| Round won | Ascending victory tune |
| Game start countdown | Beep per second; higher pitch on go |

⚠️ Use a passive buzzer, not an active buzzer. Active buzzers produce only one frequency. Passive buzzers are driven by PWM from the ESP32 and can play tones using tone().

# 10. Silent Mode

Silent mode is a 2-minute stealth mechanic designed for night play. It makes the player completely invisible and inaudible: no LEDs, dimmed display. Hit feedback (vibration) still works so the player knows they have been hit.

| Setting | Behaviour |
| --- | --- |
| Duration | 2 minutes; auto-deactivates |
| LEDs | All off: including team colour and sensors |
| Display | Dimmed to minimum brightness |
| Display VCC switch | Independent physical switch: fully cuts OLED power |
| Vibration on hit | Unchanged: still fires normally |
| Sound on hit | No sound on hit, even outside silent mode |
| Activate sound | Short descending tone before going silent |
| Deactivate sound | Short ascending tone on restore |
| Team colour on deactivate | Restores to team colour immediately |

💡 Silent mode is only useful at night. During the day, 0% and 5% LED brightness are visually identical outdoors. It is intentionally an all-or-nothing stealth tool.

# 11. Build Order

## Phase 1: Single Unit Hardware

Get one complete unit working before building four.

- Wire ESP32 to OLED (I2C: SDA, SCL, VCC, GND). Display Hello World.

- Wire passive buzzer to GPIO. Play a tone.

- Wire vibration motor via transistor. Fire a pulse.

- Wire one TSOP38238 IR receiver. Detect IR signal from TV remote to confirm it works.

- Wire IR LED via 2N2222 transistor. Transmit a test signal and confirm reception on the receiver.

- Build the 32-bit IR packet. Transmit from one ESP32, receive and parse on another.

- Add remaining 4 IR receivers. Confirm all 5 are detected individually.

- Wire WS2812B LEDs. Confirm chain responds to team colours.

- Wire all buttons and potentiometer. Confirm readings.

- Wire TP4056 + 18650. Confirm charging and stable power.

- Assemble cord with all 7 wires. Test end-to-end with vest sensors and LEDs.

## Phase 2: Networking & Game Logic

- Get ESP-NOW broadcasting between two units. Send a test packet; confirm receipt.

- Implement SYNC_REQUEST / SYNC_RESPONSE. Confirm table sync between two nodes.

- Implement HIT_CLAIM and SHOT_CONFIRM. Confirm two-factor hit registration works.

- Add version vector logic. Confirm rebooted node gets state restored correctly.

- Implement lobby: team select, ready vote, game start.

- Implement win condition check after every HP change.

- Implement reset vote.

- Build and test with all 4 units.

## Phase 3: Authentication

- Integrate uECC library. Generate keypair on boot and print public key to serial to confirm.

- Implement PUBKEY_BROADCAST. Confirm all nodes collect each other's public keys during lobby.

- Implement shared secret derivation. Confirm two nodes derive an identical secret.

- Add HMAC field to all packets. Confirm nodes reject packets with bad HMAC.

- Test reboot eviction: reboot one node mid-game, confirm its packets are rejected and LEDs flash.

- Test replay attack prevention with used-nonce log.

- Full 4-player test with all auth enabled.

# 12. Arduino IDE Setup

## 12.1 Board and Core

- Board: ESP32 Dev Module (or the specific variant in use)

- Arduino ESP32 core: v3.3.5 (use exactly this: ESP-NOW callback signatures changed in v3.x)

- Install via Boards Manager: search "esp32 by Espressif"

## 12.2 Libraries Required

| Library | Purpose | Install Via |
| --- | --- | --- |
| ESP-NOW | Built into arduino-esp32 core | Included |
| Preferences | Flash storage for game table | Included |
| Wire | I2C for OLED | Included |
| Adafruit SSD1306 | OLED display driver | Library Manager |
| Adafruit GFX | Required by SSD1306 | Library Manager |
| FastLED or Adafruit NeoPixel | WS2812B LED control | Library Manager |
| IRremoteESP8266 | IR send and receive | Library Manager |
| uECC | ECDH key exchange (Phase 3) | Manual install from GitHub |

## 12.3 Key ESP-NOW Notes for Core v3.x

- Callback signatures changed in v3.x: use the new signatures or compilation will fail

- Zero-initialise the peer struct before use: memset(&peerInfo, 0, sizeof(peerInfo))

- Register broadcast peer (FF:FF:FF:FF:FF:FF) explicitly before first broadcast

- readStringUntil() returns void in v3.x: use a manual char buffer instead

// Correct v3.x send callback signature

void onDataSent(const uint8_t *mac, esp_now_send_status_t status) { ... }

// Correct v3.x receive callback signature

void onDataRecv(const esp_now_recv_info_t *info, const uint8_t *data, int len) { ... }

# 13. Testing Checklist

## Single Unit

- OLED displays correctly

- Buzzer plays tones

- Vibration motor fires

- All 5 IR receivers trigger individually

- IR transmitter fires and is received by another unit

- WS2812B chain shows all team colours

- All buttons register

- Potentiometer reads across full range

- Battery charges and powers unit cleanly

- Cord: all 7 lines confirmed end to end

## Two Units

- ESP-NOW packets received reliably

- Table sync works: change on one, sync to other

- Hit claim + shot confirm: legitimate hit commits

- Fake hit claim without matching confirm: rejected after 2s

- Fake confirm without matching claim: rejected after 2s

- Reboot mid sync: rebooted node gets correct state back

## Four Units: Full Game

- Lobby: all 4 teams assigned, ready votes, game starts

- Hits register correctly across all pairs

- Friendly fire correctly ignored

- Win condition triggers correctly for 1v1v1v1 and 2v2

- Reset vote: all 4 must vote

- Silent mode: player invisible to others at night

- Reboot eviction: rebooted player LEDs flash, packets rejected

# 14. Anti-Cheat Summary

| Threat | Defence |
| --- | --- |
| Reboot to reset HP | Version 0 on reboot; majority pushes correct state back; ECDH eviction |
| Broadcast fake high HP | Positive delta rejected by all nodes |
| Replay old valid packet | Used-nonce log; version must be exactly current+1 |
| Fake hit claim | Shooter never confirms without matching nonce; pending expires |
| Fake shot confirm | No matching HIT_CLAIM on other nodes; pending expires |
| Forged packet | HMAC with ECDH session key rejects it |
| Missed hit packet | HIT_CLAIM retransmitted 3×; close range makes loss negligible |
| Evicted node detected | All LEDs flash rapidly; buzzer sounds |

# 15. Future Extensions

## 15.1 Repeater Network

For scenarios requiring range beyond 200m (large outdoor areas, multi-building deployments), ESP32 repeater nodes can extend coverage without modifying player node code. Repeaters listen for ESP-NOW broadcasts and forward them via a painlessMesh network between repeaters, then re-broadcast locally to player nodes. Player nodes are unaware of the repeater layer.

💡 Repeaters need only one addition: a seen-nonce deduplication buffer to prevent packets looping through the mesh indefinitely.

## 15.2 Additional Weapon Modes

The potentiometer can support more than two modes. Sniper (high damage, single shot, long cooldown), shotgun (low damage per hit, rapid), and shield modes are all achievable by expanding the weapon table and adding mode labels to the display.

## 15.3 Spectator Node

A 5th ESP32 board can act as a spectator display: it receives all ESP-NOW traffic and renders a live scoreboard on a larger screen, with no HP slot and no hit detection. No code changes are needed on player nodes.

## 15.4 Post-Game Stats

Stats are not stored in the current design by choice. To add kill/death logging, a circular buffer can be written to RAM during the game and dumped via serial at round end.

⚠️ Do not write stats to Preferences mid-game. Excessive flash writes degrade it over time.